Continuous Pipe Rolling Mill 

Continuous Pipe Rolling Mill was designed for the production of seamless, steel tubes with diameters; 

· ranging from 21,3 mm  to 159 mm including, wall thickness from 2,3 mm to 25 mm and lengths from 6 to 16 meters,

 of the following types:

· line and construction pipes,

· installation pipes,

· boiler tubes (grade: I – III),

· pipes for cold working,

· pipes for further processing into drill pipes, 

made of straight carbon, medium-alloy and low-alloy steel. Hot-rolled, seamless, steel tubes were to meet the standards: PN, DIN, ASTM, API5, CT, API5L, BS, GOST. Continuous Pipe Rolling Mill was scheduled for an annual production capacity of 150 000 tons.

The pipe manufacturing process according to MRK-AR technology consists of rolling of thick-walled tubular blanks on a cylindrical mandrel which is moving at a controlled speed rate along the rolling axis. At the end of rolling process charging tube is removed from a mandrel. It’s a modern solution applied in tube rolling mills of high productivity and for much larger size ranges of rolled tubes. Due to the use of a continuous mandrel rolling mill the rolling process is limited to pipes with diameter of less than 244,5 mm. 

The technological line is composed of the following devices:

· rotary heating furnace,

· walking beam furnace,

· rotary piercing mill,
· mill reducer bushings, 

· continuous mandrel mill,  

· mandrel hoist, 
· pipe induction reheating unit, 

· stretch reducing rolling mill,

· walking beam cooling bed,

· End finishing line No.1 for tubes with diameters: 21,3 – 114,3 mm, wall thickness: 2,3 – 12,5mm, with lengths from 6 to 12 meters

· End finishing line No.2 for tubes (as above the line No.1)

· End finishing line No.3 for tubes with diameters: 44,5 – 159,0 mm, wall thickness: 4,5 – 25,0mm, with lengths from 6 to 16 meters.

SAMPLE PHOTOS OF THE TECHNOLOGICAL LINE

[image: image1.png]



[image: image2.png]PIEC OBROTOWY - WSADZARKA | WYSADZARKA





[image: image3.png]



[image: image4.png]



[image: image5.png]TRANSPORT MIEDZY WALCARKA GIAGEA | REDUKCYINA





[image: image6.png]



[image: image7.png]



[image: image8.png]WALCARKA-SKOSNA





[image: image9.png]



[image: image10.png]4 3 =i 4
LINIA WYKANCZAJACANR 1" I





[image: image11.png]



[image: image12.png]HALA NR 10 - SKLADOWISKA WSADU | PItA DO CIE!






